## 1. Purpose

This SOP establishes a structured Permit to Work (PTW) system to control high-risk work activities, ensuring safety and regulatory compliance. It defines the responsibilities and procedures for issuing, authorizing, and monitoring work permits in hazardous environments.

## 2. Scope

Applies to all employees, contractors, and visitors performing high-risk tasks, including:

- Hot Work (welding, cutting, grinding)

- Confined Space Entry

- Work at Heights

- Electrical Work

- Excavations

- Hazardous Chemical Handling

- Lockout/Tagout (LOTO)

## 3. References and Compliance Requirements

### 3.1 South African Legislation

- Occupational Health and Safety Act (OHSA) No. 85 of 1993

  - Section 8(1): Employers must provide a safe working environment.

  - Section 10: Prohibits hazardous work without proper control measures.

- Construction Regulations (2014)

  - Regulation 10: Work at heights must have a fall protection plan.

  - Regulation 29: Requires a Permit to Work (PTW) system for hazardous activities.

- General Safety Regulations (Regulation 9)

  - Requires a permit for work in confined spaces.

- Mine Health and Safety Act No. 29 of 1996, Section 10

  - Mandates risk assessments and safety measures for high-risk mining operations.

### 3.2 International Standards

- ISO 45001:2018 (Clause 8.1.2)

  - Requires control of high-risk activities using authorization systems.

- ISO 14001:2015 (Clause 8.2)

  - Ensures environmental hazards are controlled through permit systems.

- NFPA 51B (Standard for Fire Prevention During Welding, Cutting, and Grinding)

  - Defines hot work permit requirements.

- OSHA 29 CFR 1910.146 (Confined Spaces Standard)

  - Requires a permit system for confined space entry.

- ILO Code of Practice on Occupational Safety and Health (2001)

  - Recommends a structured permit system for hazardous work.

## 4. Definitions

- Permit to Work (PTW): A formal written authorization for high-risk work.

- Permit Issuer: A competent person authorized to approve work permits.

- Permit Holder: The person in charge of the work (e.g., contractor or supervisor).

- Worksite Supervisor: Responsible for monitoring work compliance.

- LOTO (Lockout/Tagout): A safety procedure to isolate energy sources before maintenance.

## 5. Responsibilities

| Role | Responsibilities |
| --- | --- |
| Management | Implement and enforce the PTW system, provide training, and conduct audits. |
| HSE Officer | Ensure compliance with PTW requirements and conduct safety inspections. |
| Permit Issuer | Review and approve permits, ensuring proper safety controls. |
| Permit Holder | Ensure workers comply with safety requirements and return permits after completion. |
| All Employees | Follow permit conditions and report unsafe conditions immediately. |

## 6. Permit to Work Categories and Requirements

### 6.1 Hot Work Permit (Welding, Cutting, Grinding)

- Fire Watch must be assigned.

- Fire extinguishers and fire blankets must be available.

- Work must be 5 meters away from flammable materials.

- A gas detector must be used in hazardous areas.

### 6.2 Confined Space Entry Permit

- Risk assessment must be conducted.

- Gas testing for O₂, CO₂, and toxic gases is required.

- Trained standby personnel must be available.

- Rescue equipment (harness, lifeline) must be ready.

### 6.3 Work at Heights Permit

- Workers must wear fall protection equipment (harness, lanyards).

- Guardrails or lifelines must be in place.

- A pre-work weather check must be conducted.

### 6.4 Electrical Work Permit

- LOTO procedures must be followed.

- Only qualified electricians can perform electrical work.

- Insulated gloves and tools must be used.

### 6.5 Excavation Permit

- Underground utility maps must be reviewed before digging.

- Shoring or trench boxes must be used for deep excavations.

- Barricades and signage must be placed around the area.

### 6.6 Hazardous Chemical Handling Permit

- Safety Data Sheets (SDS) must be available.

- Respiratory protection must be used for toxic chemicals.

- Chemical storage and disposal must follow ISO 14001 standards.

## 7. Permit to Work Procedure

### 7.1 Permit Issuance

- The Permit Holder must submit a Permit to Work request form.

- The Permit Issuer reviews the application and conducts a pre-work risk assessment.

- The permit is issued only if all controls are in place.

### 7.2 Work Execution and Monitoring

- The work team must sign the permit before starting.

- The Permit Holder supervises the work.

- The HSE Officer conducts periodic safety checks.

### 7.3 Permit Closure and Handover

- After work completion, the Permit Holder returns the permit to the Issuer.

- The worksite is inspected for safety before permit closure.

- All safety measures (e.g., LOTO, barricades) are removed.

## 8. Training Requirements

- Permit Issuers and Holders must receive annual PTW training.

- Workers must be trained in hazard identification for each permit type.

- Emergency response drills for high-risk work must be conducted bi-annually.

## 9. Documentation and Record-Keeping

| Document | Retention Period |
| --- | --- |
| Issued Permits | 3 years |
| Risk Assessments | 5 years |
| Incident Reports | 10 years |
| Training Records | 5 years |

## 10. Review and Continuous Improvement

- The PTW system must be audited annually.

- Non-compliance incidents must be investigated and corrective actions implemented.

## 11. Non-Compliance and Disciplinary Action

- Unauthorized work without a permit will result in disciplinary action.

- Failure to comply with PTW conditions may lead to suspension or dismissal.

## 12. References

### 12.1 South African Legislation

- Occupational Health and Safety Act No. 85 of 1993 – Sections 8 and 10.

- Construction Regulations (2014) – Regulations 10, 29.

- General Safety Regulations – Regulation 9 (Confined Space Work).

- Mine Health and Safety Act No. 29 of 1996 – Section 10.

### 12.2 International Standards and Best Practices

- ISO 45001:2018 – Clause 8.1.2 (Control of High-Risk Work).

- ISO 14001:2015 – Clause 8.2 (Environmental Controls).

- NFPA 51B – Hot Work Safety.

- OSHA 29 CFR 1910.146 – Confined Spaces Standard.

- ILO Code of Practice (2001) – Permit to Work Systems.